Robotic Arm Project

#include <Servo.h>

// --- Servo Motor Definitions ---

Servo baseServo;

Servo shoulderServo;

Servo elbowServo;

const int BASE_PIN = 9;

const int SHOULDER_PIN = 10;

const int ELBOW_PIN = 11;

// --- Input/Control Definitions ---

const int POT_BASE = A0;

const int POT_SHOULDER = A1;

const int POT_ELBOW = A2;

const int RECORD_BUTTON_PIN = 2;

const int PLAY_BUTTON_PIN = 3;

// --- Recording Array and Variables ---

#define MAX_POSITIONS 100 // Maximum number of positions to record

// Array to store the servo angles (3 columns for 3 servos)

int recordedPositions[MAX_POSITIONS][3];

int currentPositionIndex = 0;

bool isRecording = false;

// --- Timing ---

const int RECORD_INTERVAL_MS = 100; // Record a position every 100 milliseconds

unsigned long lastRecordTime = 0;

void setup() {

Serial.begin(9600);

// Attach servos to their pins

baseServo.attach(BASE_PIN);

shoulderServo.attach(SHOULDER_PIN);

elbowServo.attach(ELBOW_PIN);

// Initialize button pins

pinMode(RECORD_BUTTON_PIN, INPUT_PULLUP);

pinMode(PLAY_BUTTON_PIN, INPUT_PULLUP);

// Move all servos to a starting position (e.g., 90 degrees)

baseServo.write(90);

shoulderServo.write(90);

elbowServo.write(90);

Serial.println("Robotics Arm Ready. Press RECORD button to start.");

}

void loop() {

// Check for button presses and handle debouncing (simplified)

handleButtonPresses();

if (isRecording) {

recordPositions();

} else {

// When not recording, the potentiometers control the arm manually

manualControl();

}

}

/**

* @brief Handles the logic for the Record and Play buttons.

*/

void handleButtonPresses() {

// --- RECORD BUTTON Logic ---

if (digitalRead(RECORD_BUTTON_PIN) == LOW) {

// Debounce delay

delay(200);

if (!isRecording) {

// Start Recording

currentPositionIndex = 0; // Reset index for a new recording

isRecording = true;

Serial.println("--- RECORDING STARTED ---");

} else {

// Stop Recording

isRecording = false;

Serial.print("--- RECORDING STOPPED. Total Frames: ");

Serial.print(currentPositionIndex);

Serial.println(" ---");

}

// Wait for button release

while(digitalRead(RECORD_BUTTON_PIN) == LOW);

}

// --- PLAY BUTTON Logic ---

if (digitalRead(PLAY_BUTTON_PIN) == LOW) {

// Debounce delay

delay(200);

// Only play if not currently recording and we have recorded frames

if (!isRecording && currentPositionIndex > 0) {

Serial.println("--- PLAYBACK STARTED ---");

playbackPositions();

Serial.println("--- PLAYBACK FINISHED ---");

}

// Wait for button release

while(digitalRead(PLAY_BUTTON_PIN) == LOW);

}

}

/**

* @brief Reads potentiometers and moves the servos manually.

*/

void manualControl() {

int baseAngle = map(analogRead(POT_BASE), 0, 1023, 0, 180);

int shoulderAngle = map(analogRead(POT_SHOULDER), 0, 1023, 0, 180);

int elbowAngle = map(analogRead(POT_ELBOW), 0, 1023, 0, 180);

baseServo.write(baseAngle);

shoulderServo.write(shoulderAngle);

elbowServo.write(elbowAngle);

}

/**

* @brief Saves the current servo positions into the array at a set interval.

*/

void recordPositions() {

if (currentPositionIndex < MAX_POSITIONS) {

if (millis() - lastRecordTime >= RECORD_INTERVAL_MS) {

// Read current angles (which are being set by manualControl)

int baseAngle = map(analogRead(POT_BASE), 0, 1023, 0, 180);

int shoulderAngle = map(analogRead(POT_SHOULDER), 0, 1023, 0, 180);

int elbowAngle = map(analogRead(POT_ELBOW), 0, 1023, 0, 180);

// Store the angles

recordedPositions[currentPositionIndex][0] = baseAngle;

recordedPositions[currentPositionIndex][1] = shoulderAngle;

recordedPositions[currentPositionIndex][2] = elbowAngle;

currentPositionIndex++;

lastRecordTime = millis();

}

} else {

// Stop recording if the array is full

isRecording = false;

Serial.println("!!! Array Full! Recording Stopped Automatically !!!");

}

}

/**

* @brief Replays the saved servo positions sequentially.

*/

void playbackPositions() {

for (int i = 0; i < currentPositionIndex; i++) {

int baseAngle = recordedPositions[i][0];

int shoulderAngle = recordedPositions[i][1];

int elbowAngle = recordedPositions[i][2];

baseServo.write(baseAngle);

shoulderServo.write(shoulderAngle);

elbowServo.write(elbowAngle);

// Wait for the same interval used during recording for smooth playback

delay(RECORD_INTERVAL_MS);

}

}