IOT-Enabled Robot

1. Hardware Connections

1.1 Core Components

- ESP32 (main control board)

- L298N Motor Driver (for 4 motors)

- 12V Battery Pack (for motors and power)

- 16x2 LCD Display (status display)

1.2 Sensors

- HC-SR04 Ultrasonic Sensor (for obstacle detection)

- 3 IR Sensors (for line following)

- Battery Voltage Sensor (monitor battery status)

- Optional MPU6050 (for balancing)

1.3 Detailed Connections

1. Motor Driver (L298N)

| Pin | ESP32 Pin | Function |
| --- | --- | --- |
| ENA | GPIO 25 | Speed control for Motor A |
| ENB | GPIO 26 | Speed control for Motor B |
| IN1 | GPIO 32 | Motor A Direction 1 |
| IN2 | GPIO 33 | Motor A Direction 2 |
| IN3 | GPIO 27 | Motor B Direction 1 |
| IN4 | GPIO 14 | Motor B Direction 2 |
| VCC | Battery+ | Motor driver power supply |
| GND | Battery- | Common ground |

2. Ultrasonic Sensor (HC-SR04)

| Pin | ESP32 Pin |
| --- | --- |
| Trigger | GPIO 4 |
| Echo | GPIO 5 |
| VCC | 5V |
| GND | GND |

3. IR Sensors

| Sensor | ESP32 Pin |
| --- | --- |
| Left | GPIO 34 |
| Center | GPIO 35 |
| Right | GPIO 39 |

4. ESP32 Communication (Wi-Fi + Bluetooth)

| Pin | ESP32 Pin | Function |
| --- | --- | --- |
| RX | GPIO 16 | UART communication (optional) |
| TX | GPIO 17 | UART communication (optional) |

5. LCD Display (I2C 16x2)

| Pin | ESP32 Pin |
| --- | --- |
| SDA | GPIO 21 |
| SCL | GPIO 22 |
| VCC | 3.3V |
| GND | GND |

6. LEDs and Buzzer

- Status LEDs: Connect to GPIO pins as needed.

- Buzzer: Connect to GPIO 23 for alerts.

2. Software Code

Here's the Arduino Sketch for the ESP32 to control the robot.

Libraries Required

- Install LiquidCrystal_I2C for LCD.

- Install WiFi for ESP32 networking.

- Install NewPing for the ultrasonic sensor.

Code:-

#include <WiFi.h>

#include <LiquidCrystal_I2C.h>

#include <NewPing.h>

// WiFi Credentials

const char* ssid = "Your_SSID";

const char* password = "Your_PASSWORD";

// Ultrasonic Sensor

#define TRIG_PIN 4

#define ECHO_PIN 5

#define MAX_DISTANCE 200

NewPing sonar(TRIG_PIN, ECHO_PIN, MAX_DISTANCE);

// Motor Pins

#define ENA 25

#define ENB 26

#define IN1 32

#define IN2 33

#define IN3 27

#define IN4 14

// Line Follower Pins

#define IR_LEFT 34

#define IR_CENTER 35

#define IR_RIGHT 39

// LCD Display

LiquidCrystal_I2C lcd(0x27, 16, 2);

// Modes

enum Modes { MANUAL, AUTONOMOUS, LINE_FOLLOW };

Modes currentMode = MANUAL;

// WiFi Server

WiFiServer server(80);

// Function Declarations

void moveForward();

void moveBackward();

void turnLeft();

void turnRight();

void stopMotors();

void handleClient();

void lineFollowingMode();

void autonomousMode();

void setup() {

// Motor Pins Setup

pinMode(ENA, OUTPUT);

pinMode(ENB, OUTPUT);

pinMode(IN1, OUTPUT);

pinMode(IN2, OUTPUT);

pinMode(IN3, OUTPUT);

pinMode(IN4, OUTPUT);

// IR Sensor Pins

pinMode(IR_LEFT, INPUT);

pinMode(IR_CENTER, INPUT);

pinMode(IR_RIGHT, INPUT);

// Initialize LCD

lcd.begin();

lcd.backlight();

lcd.print("Starting Robot...");

// Connect to WiFi

WiFi.begin(ssid, password);

while (WiFi.status() != WL_CONNECTED) {

delay(1000);

lcd.setCursor(0, 1);

lcd.print("Connecting...");

}

server.begin();

lcd.clear();

lcd.print("Connected!");

}

void loop() {

handleClient();

if (currentMode == LINE_FOLLOW) {

lineFollowingMode();

} else if (currentMode == AUTONOMOUS) {

autonomousMode();

}

}

// Motor Control

void moveForward() {

digitalWrite(IN1, HIGH);

digitalWrite(IN2, LOW);

digitalWrite(IN3, HIGH);

digitalWrite(IN4, LOW);

analogWrite(ENA, 200);

analogWrite(ENB, 200);

}

void moveBackward() {

digitalWrite(IN1, LOW);

digitalWrite(IN2, HIGH);

digitalWrite(IN3, LOW);

digitalWrite(IN4, HIGH);

analogWrite(ENA, 200);

analogWrite(ENB, 200);

}

void stopMotors() {

digitalWrite(IN1, LOW);

digitalWrite(IN2, LOW);

digitalWrite(IN3, LOW);

digitalWrite(IN4, LOW);

}

// Handle WiFi Client

void handleClient() {

WiFiClient client = server.available();

if (client) {

String request = client.readStringUntil('\r');

client.flush();

if (request.indexOf("/forward") != -1) moveForward();

if (request.indexOf("/backward") != -1) moveBackward();

if (request.indexOf("/stop") != -1) stopMotors();

if (request.indexOf("/mode/line") != -1) currentMode = LINE_FOLLOW;

if (request.indexOf("/mode/auto") != -1) currentMode = AUTONOMOUS;

client.print("HTTP/1.1 200 OK\r\nContent-Type: text/html\r\n\r\n");

client.print("<h1>ESP32 Robot</h1>");

client.stop();

}

}

// Line Following Mode

void lineFollowingMode() {

int left = digitalRead(IR_LEFT);

int center = digitalRead(IR_CENTER);

int right = digitalRead(IR_RIGHT);

if (center == HIGH) moveForward();

else if (left == HIGH) turnLeft();

else if (right == HIGH) turnRight();

else stopMotors();

}

// Autonomous Mode

void autonomousMode() {

int distance = sonar.ping_cm();

if (distance < 10) moveBackward();

else moveForward();

}