Fire Fighting Bot Project

This project involves designing an autonomous robot that can detect fire, navigate towards the flame, and extinguish it using a water-sprinkler mechanism. The robot uses a flame sensor to detect fire, IR sensors for obstacle detection, and an Arduino Uno as the main controller. A servo motor is used to aim and spray water, while an L293D motor driver controls robot movement.

Code :-

#include <Servo.h>

Servo nozzle;

int flame = A0;

int leftIR = 2;

int rightIR = 3;

int pump = 9;

int ENA = 5;

int IN1 = 6;

int IN2 = 7;

int ENB = 10;

int IN3 = 11;

int IN4 = 12;

void setup() {

nozzle.attach(8);

pinMode(flame, INPUT);

pinMode(leftIR, INPUT);

pinMode(rightIR, INPUT);

pinMode(pump, OUTPUT);

pinMode(ENA, OUTPUT);

pinMode(IN1, OUTPUT);

pinMode(IN2, OUTPUT);

pinMode(ENB, OUTPUT);

pinMode(IN3, OUTPUT);

pinMode(IN4, OUTPUT);

digitalWrite(pump, LOW);

}

void forward() {

digitalWrite(IN1, HIGH);

digitalWrite(IN2, LOW);

digitalWrite(IN3, HIGH);

digitalWrite(IN4, LOW)

}

void stopBot() {

digitalWrite(IN1, LOW);

digitalWrite(IN2, LOW);

digitalWrite(IN3, LOW);

digitalWrite(IN4, LOW);

}

void loop() {

int flameVal = analogRead(flame);

if (flameVal < 400) {  // fire detected

stopBot();

nozzle.write(90);    // point nozzle

digitalWrite(pump, HIGH); // spray water

delay(3000);

digitalWrite(pump, LOW);

}

else {

// No fire – normal movement

if (digitalRead(leftIR) == LOW) {

// obstacle left → turn right

digitalWrite(IN1, HIGH);

digitalWrite(IN2, LOW);

digitalWrite(IN3, LOW);

digitalWrite(IN4, HIGH);

}

else if (digitalRead(rightIR) == LOW) {

// obstacle right → turn left

digitalWrite(IN1, LOW);

digitalWrite(IN2, HIGH);

digitalWrite(IN3, HIGH);

digitalWrite(IN4, LOW);

}

else {

forward();

}

}

}